LICENSE PROFICIENCY CHECK

S.I.M.

2008

TRAINEES

Name:______________________________  Pos:____________

Training Dates :		GND:______________

SIM:_______________

1. PRESENTATION	2

2.USEFUL PHONE NUMBERS	3

3.ANNUAL PROGRAM	4

1.ALL WEATHER OPERATIONS	4

2.LICENSE PROFICIENCY CHECK (LPC)	4

3.RECURRENT TRAINING	5

1.SYSTEMS REVIEWED IN 2008	5

2.TRAINING GRADING	5

3.PROFICIENCY CHECK GRADING (OPC AND LPC).	5

4.ABBREVIATIONS	5

4.SIMULATOR TRAINING 2008	6

1.SIMULATOR TRAINING 2008 (Page 1)     BILBAO	6

2.SIMULATOR TRAINING 2008 (Page 2)     BILBAO	7

5.SIMULATOR LPC 2008	8

1.SIMULATOR LPC 2008 (Page 1)     BILBAO – PAU  + AWO	8

2.SIMULATOR LPC 2008 (Page 2)     PAU	9

6. JEPPESEN CHARTS NEEDED	10

1.TRAINING and LPC:	10

2.LPC:	10

7.FLIGHT PLANS	11

1.LPC 2008      BILBAO – PAU	11

# PRESENTATION

This training is performed for pilots who do not need to be trained under TRTO. Nevertheless, it respects entirely the JAR standards.

This training contains the simulator program for LPC.

# USEFUL PHONE NUMBERS

Jean Carcassonne (SFI S.I.M)............…………………..	0033 6 09 04 06 01

0033 1 46 02 34 62

Gwenael Lesourd (TRE S.I.M.)…………………….....…	0033 6 80 85 59 86

Robert Gallet (TRE S.I.M).........................................	0036 6 13 24 04 60

Aristide Barbaras (TRE S.I.M.)..........................................	0033 6 86 43 95 72

Hotel Kyriad Prestige – Le Bourget……………………..	0033 1 49 34 10 38

Hotel Park Inn – Charles De Gaulle	0033 1 30 18 21 00

Hotel Novotel – Le Bourget.............................................	0033 1 48 67 48 88

Air Hotel – Le Bourget.............................................	0033 1 48 67 20 37

Flight Safety – Le Bourget.............................................	0033 1 49 92 19 19

# ANNUAL PROGRAM

## ALL WEATHER OPERATIONS

If needed, it remains possible to renew AWO. The following maneuvers are compulsory.

  - Rejected Take-off at Minimum RVR

  - Autoland Approach,

  - Go-around at DH,

  - Normal landing.

## LICENSE PROFICIENCY CHECK (LPC)

The form JAR-FCL 1.240 must be filled by the instructor to renew pilots Fokker 100 Type Rating and Instrument Rating.

The following items are mandatory for LPC, only (X) are requested for OPC

1.4 	Pre-flight checks and radio,

1.6 	Before take-off checks,

2.5.2	Engine failure after V1 (B),

2.6	Rejected Take-Off (A),

3.4	Minimum 2 (or 1) abnormal system operations,

3.6	Minimum 1 (or 2) abnormal and emergency procedures,

3.9.1	Adherence to SID and STAR,

3.9.3	Manual single engine Precision approach (C),

3.9.4	Non precision approach to minimums (D),

4.3	Manual single engine go-around (E),

5.5	Single engine landing (F).

# RECURRENT TRAINING

## SYSTEMS REVIEWED IN 2008

If needed, in 2008 training is oriented to use the following systems :

FIRE PROTECTION

WARNING AND CAUTION

APU

HYDRAULIC SYSTEM

TCAS

LANDING GEAR AND BRAKES

AUTOFLIGHT

## TRAINING GRADING

Grading:

DONE = Exercise is normally completed.

REDO = Exercise has to be redone due to a low performance.

## PROFICIENCY CHECK GRADING (OPC AND LPC).

Grading:

+	 Standard

-	Below Standard (Extra training needed).

## ABBREVIATIONS

AEO = All Engines Operating,

OEI = One Engine Inoperative,

NPA = Non Precision Approach.

# SIMULATOR TRAINING 2008

## SIMULATOR TRAINING 2008 (Page 1)     BILBAO

| Briefing | CMD PF, BIO RWY 30, Flaps 8° for all Take-off, Visual DAY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| State | Engines stopped Version SIM-IRS Flight n° 406 |  |  |  |  |  |  |  |
| ATIS DEP | “ X ” 24018KT 1500m SKC T° 12/08 Q1024 = |  |  | ATIS ARR | ILS 30 “ X ” 24018KT 1500m SKC T° 12/08 Q1024 = |  |  |  |
| Initial Position | Any town around A 1 |  |  | Route | DEP RWY 30 MAPAX 1A ARR RWY 30 MAPAX 1J |  |  |  |
| Settings | ZFM (T) | FUEL (T) | TOM (T) | Cost Index | PAX | % MAC | CRZ ALT | CRZ WIND |
|  | 34.6 | 5.0 | 39.4 | 50 | 90 | 26 | FL 80 | 270/35 |
| ATC | BIO GND TWR APP ATIS 121.70 118.50 120.70 118.82 |  |  |  |  |  |  |  |

| RanK | PF | ExerciSe | TIME |  | DONE | REDO |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | INS | Simulator Safety Briefing to be completed | 0.00 | 0.00 |  |  |
| 1 | CMD | MEL : BOTH FMS US (FMC1and FMC2 FAIL) (MEL 34-61-1 : OM B 09-34 page 16) Leave both FMS US for the whole training. | 0.05 | 0.05 |  |  |
| 2 | « | Flight Preparation – Check lists | 0.15 | 0.20 |  |  |
| 3 | « | HUNG START (Auto reset failure after action) | 0.10 | 0.30 |  |  |
| 4 | « | Taxi to RWY 30 Tango 1 and Charlie 1 Clearance MAPAX [1A] FL 80 Squawk 3615 | 0.05 | 0.35 |  |  |
| 5 | « | BLOWN TIRE at 110 Kt REJECTED TAKE-OFF | 0.05 | 0.40 |  |  |
| 6 | « | Reset failure. Normal Take-Off | 0.10 | 0.50 |  |  |
| 7 | « | SID MAPAX 1A | 0.00 | 0.50 |  |  |
| 8 | « | At FL 80 TCAS RA | 0.10 | 1.00 |  |  |
| 9 | « | AP1 and AP 2 FAIL | 0.05 | 1.05 |  |  |
| 10 | « | At MAPAX : STD Arrival MAPAX 1J | 0.05 | 1.10 |  |  |
| 11 | « | Standard ILS 30 Manual | 0.15 | 1.25 |  |  |
| 12 | « | Landing. (Reset all failures) | 0.05 | 1.30 |  |  |
| 13 | « | Normal take-off. Clearance 5000 ft then radar vectors. | 0.10 | 1.40 |  |  |
| 14 | « | After T/O, ANTI RETRACTION SOLENOID FAIL (Prepare Malf. Before T/O and reset after action) | 0.05 | 1.45 |  |  |
| 15 | « | Localizer RWY 30 (Off-screen kill : IBL-GS) | 0.10 | 1.55 |  |  |
| 16 | « | Normal Landing (Reset Failures) | 0.05 | 2.00 |  |  |

## SIMULATOR TRAINING 2008 (Page 2)     BILBAO

| Briefing | COP PF, BIO RWY 30, Flaps 8° for all Take-off, Visual DAY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| State | Engines running Version SIM-IRS Flight n° 407 |  |  |  |  |  |  |  |
| ATIS DEP | “ X ” 24018KT 1500m SKC T° 12/08 Q1034 = |  |  | ATIS ARR | ILS 30 “ X ” 24018KT 1500m SKC T° 12/08 Q1034 = |  |  |  |
| Initial Position | Threshold RWY 30 |  |  | Route | 6000’ ALBIX |  |  |  |
| Settings | ZFM (T) | FUEL (T) | TOM (T) | Cost Index | PAX | % MAC | CRZ ALT | CRZ WIND |
|  | 34.6 | 5.0 | 39.4 | 50 | 90 | 26 | 5000 | 270/35 |
| ATC | BIO GND TWR APP ATIS 121.70 118.50 120.70 118.82 |  |  |  |  |  |  |  |

| RanK | pf | ExerciSe | TIME |  | DONE | REDO |
| --- | --- | --- | --- | --- | --- | --- |
| 17 | COP | Rapid Flight preparation without FMS | 0.15 | 2.30 |  |  |
| 18 | « | Take-off. Clearance 6000 ft to ALBIX | 0.05 | 2.35 |  |  |
| 19 | « | After T/O SQUAT SW FAIL TO GROUND (Prepare malfunction before T/O) | 0.05 | 2.40 |  |  |
| 20 | « | VOR APP RWY 30. ILS is U/S | 0.15 | 2.55 |  |  |
| 21 | « | Normal landing RWY 30 (Reset failures) | 0.05 | 3.00 |  |  |
| 22 | « | Weather is improving to 5 Km. Visual circuits training | 0.00 | 3.00 |  |  |
| 23 | « | WINDSHEAR at Take-Off (Program Windshear n° 4 : T/O severe) | 0.05 | 3.05 |  |  |
| 24 | « | AP1 and 2 FAIL Right hand Visual circuit 1500’ | 0.05 | 3.10 |  |  |
| 25 | « | WINDSHEAR on approach Prepare windshear n° 8 PHL or 9 JFK : severe) | 0.05 | 3.15 |  |  |
| 26 | « | GO AROUND – ENG SEIZURE | 0.00 | 3.15 |  |  |
| 27 | « | OEI Visual Circuit at 1500’ by the right | 0.05 | 3.20 |  |  |
| 28 | « | Normal landing (Reset all failures) | 0.05 | 3.25 |  |  |
| 29 | CMD | WINDSHEAR at Take-Off (Same as before) | 0.05 | 3.30 |  |  |
| 30 | « | AP1 and 2 FAIL Right hand Visual circuit 1500’ | 0.05 | 3.35 |  |  |
| 31 | « | WINDSHEAR on approach (Same as before) | 0.05 | 3.40 |  |  |
| 32 | « | GO AROUND – ENG FIRE INEXTINGUISHABLE | 0.00 | 3.45 |  |  |
| 33 | « | OEI Visual Circuit at 1500’ by the right | 0.05 | 3.45 |  |  |
| 34 | « | Final landing – Evacuation (Reset all failures) | 0.05 | 3.50 |  |  |
| 35 | COP | Perform one more Circuit if time permits |  |  |  |  |

# SIMULATOR LPC 2008

## SIMULATOR LPC 2008 (Page 1)     BILBAO – PAU  + AWO

| Briefing | CMD PF, BIO RWY 30, Flaps Optimum Visual = DAY |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| State | Engines stopped Version SIM 2 IRS Flight n° 346 |  |  |  |  |  |  |  |  |
| ATIS DEP | ILS 30, LVP in force, FG, Wind calm, RVR 125m, SKC, T° 16/16, QNH 1033 |  |  | METAR ARR | ILS 31,FG, Wind 280/5 , RVR 250m, OVC 100ft, T° 12/12, QNH 1032 |  |  |  |  |
| Initial Position | Any town around A 1 |  |  | Route | SSN – BTZ – RWY 31 |  |  |  |  |
| Settings | ZFM (T) | FUEL (T) | TOM (T) | Cost Index |  | PAX | % MAC | CRZ ALT | CRZ WIND |
|  | CMD | CMD | CMD | 50 |  | 50 | 26 | FL 210 | 270/50 |
| ATC | BIO GND TWR APP BIO ATIS PUF APP TWR GND PUF ATIS 121.70 118.50 120.70 118.82 121.17 124.15 121.75 128.47 |  |  |  |  |  |  |  |  |

| RanK | PF | REF | ExerciSe | TIME |  | + | - |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | CMD | 3.4.5 | SYS 2 RH HYD PUMP FAIL (Refer to MEL) | 0.05 | 0.05 |  |  |
| 2 | « | 1.4 1.6 | Flight Preparation, Check-lists and Start | 0.20 | 0.25 |  |  |
| 3 | « | - | Taxi to RWY 30 Clearance SAN SEBASTIAN [1 B] DEP | 0.05 | 0.30 |  |  |
| 4 | « | 3.4.6 | ENGINE INDICATION PROBLEM at T/O | 0.05 | 0.35 |  |  |
| 5 | « | 3.4.0 | LVTO Take-Off | 0.00 | 0.35 |  |  |
| 6 | « | 3.9.1 | SID + HYD FAILURE | 0.10 | 0.45 |  |  |
| 7 | « | 3.6.9 | TCAS RA during climb or cruise | 0.05 | 0.50 |  |  |
| 8 | « | 3.9.1 | STAR BIARRITZ 2D | 0.10 | 1.00 |  |  |
| 9 | « | 6.2 | Radar vectors CAT III ILS 31 | 0.10 | 1.10 |  |  |
| 10 | « | 6.3 | Go-Around | 0.05 | 1.15 |  |  |
| 11 | « | 6.2 | CAT III ILS | 0.05 | 1.20 |  |  |
| 12 | « | 6.4 | Landing. Reset all failures | 0.05 | 1.25 |  |  |
| 13 | « | 6.1 A 2.6 | Rejected Take Off with RVR=125 m | 0.05 | 1.30 |  |  |
| 14 | « | 6.1 B 2.5.2 | Take Off with RVR=125 m ENG FAILURE at V1+ | 0.05 | 1.35 |  |  |
| 15 | « | C 3.9.3 3.4.8 | RWY 31 ILS CAT I RVR 600 m without AP | 0.10 | 1.40 |  |  |
| 16 | « | E 4.3 3.4.12 | At height <800 ft LEFT GEAR DOOR PROBLEM Single Engine Go-around | 0.05 | 1.45 |  |  |
| 17 | « | D 3.9.4 | Radar vectors LLZ 16 (GS U/S) | 0.10 | 1.55 |  |  |
| 18 | « | F 5.5 | Single Engine Landing | 0.05 | 2.00 |  |  |

## SIMULATOR LPC 2008 (Page 2)     PAU

| Briefing | COP PF, PUF RWY 31, Flaps Optimum, Visual = DAY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| State | Engines running Version SIM 2 IRS Flight n° 347 |  |  |  |  |  |  |  |
| ATIS DEP | “ x ” 26005KT 600m FG OVC002 12/11 Q1032 = |  |  | ATIS ARR | “ Y ” 26005KT 1600m BR OVC005 13/10 Q1031 = |  |  |  |
| Initial Position | Taxiway “NE” for RWY 31 |  |  | Route | ATC No FMS Preparation |  |  |  |
| Settings | ZFM (T) | FUEL (T) | TOM (T) | Cost Index | PAX | % MAC | CRZ ALT | CRZ WIND |
|  | 34.4 | 5.0 | 39.2 | 50 | 50 | 26 | FL 50 | 230/25 |
| ATC | PUF APP TWR GND ATIS 121.17 124.15 121.75 128.47 |  |  |  |  |  |  |  |

| RanK | PF | REF | ExerciSe | TIME |  | + | - |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 19 | COP | 1.6 | Flight Preparation and Check-lists | 0.20 | 2.20 |  |  |
| 20 | « | - | Taxi to RWY 31, Clearance TAN FL 50 Squawk 3615 | 0.05 | 2.25 |  |  |
| 21 | « | A 2.6 3.6.1 | Rejected Take-Off due to FIRE IN CABIN announced by Flight Attendant | 0.05 | 2.30 |  |  |
| 22 | « | 3.4.12 | N° 1 GROUNDSENSING SWITCH FAIL | 0.00 | 2.30 |  |  |
| 23 | « | 2.5 | Take-Off | 0.05 | 2.35 |  |  |
| 24 | « | 3.9.1 | SID + Holding if necessary | 0.10 | 2.45 |  |  |
| 25 | « | D 3.9.4 | LOCATOR DME RWY 31 | 0.10 | 2.55 |  |  |
| 26 | « |  | Normal Landing – Reset all failures | 0.05 | 3.00 |  |  |
| 27 | « | B 2.5.2 3.6.3 | Take-Off ENGINE PROBLEM at V1+ | 0.05 | 3.05 |  |  |
| 28 | « | C 3.9.3 3.4.8 | Radar vectors Single Engine ILS 31 without AP | 0.15 | 3.20 |  |  |
| 29 | « | E 4.3 | Single engine Go-Around (No visual Reference) | 0.05 | 3.25 |  |  |
| 30 | « | F 5.5 | Single Engine Landing – Reset all failures | 0.05 | 3.30 |  |  |
| 31 | « | 5.3 | Standard visual circuit with crosswind according to time remaining. | 0.20 | 3.50 |  |  |

# JEPPESEN CHARTS NEEDED

## TRAINING and LPC:

BILBAO	  	10-1P 		General

10-1P1 	General

10-2C		MAPAX Arrival RWY 30

10-3A		SAN SEBASTIAN Departure RWY 12

10-3G		MAPAX Departure RWY 30

10-9		Airport

10-9A		Parkings

11-1		ILS RWY 30

13-4		VOR RWY 30

## LPC:

PAU	 	10-1R		Radar Minimums

10-2B		BIARRITZ Arrival

10-3A		TAN Departures

10-9		Airport

10-9A		Parkings

11-1		LCTR ILS RWY 30

11-1A		CAT II LCTR ILS RWY 30

16-1		LCTR DME RWY 31

19-10		Circle-to-Land RWY 13

TOULOUSE	 	10-1P 		General

10-1P1 	General

10-1P2	General

10-1R		Radar Minimums

10-2E		TAN Arrival RWYS 14L/R

10-9		Airport

10-9A		Parkings

11-2		Approach ILS DME RWY 14R

11-2A		Final ILS DME RWY 14R

11-2B		CAT II ILS DME RWY 14R

13-2		Approach VOR DME RWY 14R

13-2A		Final VOR DME RWY 14R

# FLIGHT PLANS

## LPC 2008      BILBAO – PAU

PLAN SIM TRAINING 2008   LEBB TO LFBP    F100  LRC/F IFR 01/01/2008	PART 1

NONSTOP COMPUTED 1442Z  FOR ETD 1200Z  PROGS  2500ADF  EC-IPV    KGS

CMD. . . . . . COP. . . . . . SEN. . . . . . CAB. . . . . .

C/S SIM           EPN 000  APN .. ..   COMPANY ROUTE:  NO

SPECIAL BRIEF -

RMKS -

DEPARTURE FI . . . . .    ARRIVAL FI . . . . .

FUEL RAMP 5000 STA    Z  BLON . . . . 	LDG . . . .

EOFF . . . . STD 	  1200Z  BLOFF. . . .  T/O . . . .   ATC SLOT . . .

B/O . . . .		  TIME. . . . 		TIME . . . .

----

FUEL PLAN

GROUND DIST  139NM   AV WC M010 FL210/M16  MXSH  2/D330S

TRIP               		1438  	0.32  . . . .    MDF     1800

CONTINGENCY     		154  	0.05  . . . .

ALTERNATE          		900	0.09  . . . .    LFBO

RESERVE             		900	0.30  . . . .

MIN T/O FUEL       		3392	1.16  . . . .

TAXY/APU            		200

EXTRA               		                250	. . . .  	EXTRA FUEL

TOTAL FUEL         		3842	. . . . .  	FUEL LOADED

TOM  35312 KG  / ZFM  31670 KG    PL  6146 KG  / LMT  33674 KG

FUEL CONSUMPTION - NORMAL

WEIGHT CHANGE +1000 KG  FP   20 KG  TRIP FUEL

ATC CLEARANCE REQUESTED

LEBB   SSN 1B   BTZ   BTZ 2D   LFBP

ATC CLEARANCE ISSUED    . . . . . . . . . . . . . . . . . . . . . .

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

. . . . . . . . . . . . . . . . . . . . . . . . . . . . POB . . . .

RETURN ALTERNATE YES/NO

DEPARTURE ATIS. . . . . . . . . . . . . . . . . . . . . . . . . . .

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

PLAN SIM TRAINING 2008   LEBB TO LFBP    F100  LRC/F IFR 01/01/2008	PART 2

NONSTOP COMPUTED 1442Z  FOR ETD 1200Z  PROGS  2500ADF  EC-IPV    KGS

POSN FREQ	MT	W/V	DIST	TAS	GS	TIME ET RT AT FL

MORA        	   DTGO

LEBB                                                        					       3642

132

FEBRI	065	SID	245/023	30			008 ../../.. 040  3291 ...

102

TOC	065	092	246/025	7	354	372	001 ../../.. 210  3194 ...

95

SSN	065	092	263/025	12	353	372	002 ../../.. 098  3170 ...

117.9		      83

BTZ	076	062	280/048	13	352	377	002 ../../.. 184  2853 ...

114.15		      70

TOD	030	101	275/045	16	352	386	003 ../../.. 210  2488 ...

54

SOVOS	030	101	280/045	12	352	386	002 ../../.. 210  2689 ...

42

ORTEZ	030 	101	269/023	8			002 ../../.. 210  2574 ...

34

PU	030	101	265/016	18			005 ../../.. 065  2387 ...

16

LFBP	030	STAR	255/014	16			008 ../../.. 000  2204 ...

0

TOTAL DIST: 0132 NM   TOTAL TIME:  0032  AV WINDCOMP: P025 ISA: P011

ARR INFO: .. RWY: ..  WX: . . . . . . . . . . . . . . . . . . . . .

A/C. . . . . . CREW. . . . . .PAX. . . . FRT/BAL. . . . .MAIL. . . .

FUEL. . . . . .TRIP. . . . . .RTOM. . . . . .SPECS. . . . T/M. . . .

ETA . . . . . .

ALTERNATE	DIST	TIME	FL	TIME	MDF

LFBO		098	028	090	028	1800

....         ...    ...   ....  ...  ...    ....

ROUTE TO FIRST ALTERNATE:

ALT INFO: .. RWY: ..  WX: . . . . . . . . . . . . . . . . . . . . .

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

(FPL-GJT346-IN

-F100/M-SGHIRWY/S

-LEBB1200

-N0354F210 SSN BTZ

-LFBP0032 LFBO

-REG/ECIPV OPR/GIRJET

-E/0149 P/TBN R/ S/ J/ D/  C  A/ )

END OF FLIGHTPLAN